Network Monitoring System Report

Project Title: Network Monitoring System

Project Team

- Abhishikth Parry – Project Lead, System Configuration & Security Analysis

- Aakash Sivakumar – Intrusion Detection & Log Management

- Rajiv Bhardwaj – Data Visualization

- Sandeep Kandula – Performance Monitoring

1. Introduction

Objective:

The objective of this project is to establish a Network Monitoring System that enables real-time traffic monitoring, intrusion detection, anomaly detection, automated reporting, and data visualization. This system enhances network security, performance optimization, and proactive threat detection using industry-standard tools.

2. System Architecture

3. Tools and Technologies Used

| Feature | Tools Used | Functionality | Integration Details |
| --- | --- | --- | --- |
| Real-Time Traffic Monitoring | Suricata, Tcpdump, Wireshark | Captures and inspects live network traffic | Suricata logs traffic, Tcpdump/Wireshark used for deeper analysis |
| Intrusion Detection System (IDS) | Suricata | Detects malicious network activity | Suricata logs forwarded to Elasticsearch via Filebeat |
| Performance Metrics Analysis | Prometheus, Grafana, Node Exporter | Collects and visualizes system/network metrics | Grafana visualizes Prometheus metrics |
| Anomaly Detection | Suricata, Filebeat, Elasticsearch, Kibana | Detects unusual behavior based on traffic logs | Suricata logs sent to Elasticsearch via Filebeat, visualized in Kibana |
| Automated Reporting | Grafana, Kibana (via Elasticsearch) | Generates automated reports and alerts | Grafana alerts via webhooks, Kibana alerting being finalized |
| Data Visualization | Grafana, Kibana | Creates dashboards for logs and network data | Kibana visualizes Elasticsearch data, Grafana displays Prometheus data |

4. Implementation and Configuration

4.1 Real-Time Traffic Monitoring (Suricata, Tcpdump, Wireshark)

Step 1: Install Suricata

sudo apt update

sudo apt install -y suricata

Step 2: Configure Suricata

sudo nano /etc/suricata/suricata.yaml

Modify the af-packet section to match the correct network interface.

Step 3: Start Suricata

sudo systemctl start suricata

sudo systemctl enable suricata

Step 4: Verify Logs

sudo tail -f /var/log/suricata/eve.json

4.2 Log Forwarding with Filebeat

Step 1: Install Filebeat

sudo apt install -y filebeat

Step 2: Enable Suricata Module

sudo filebeat modules enable suricata

Step 3: Configure Filebeat for Elasticsearch Output

Edit the configuration file:

output.elasticsearch:

hosts: ["http://localhost:9200"]

setup.kibana:

host: "http://localhost:5601"

Step 4: Start Filebeat

sudo systemctl start filebeat

sudo systemctl enable filebeat

4.3 Log Storage and Visualization (Elasticsearch & Kibana)

4.4 Performance Metrics Analysis (Prometheus, Node Exporter, Grafana)

Step 1: Install Prometheus

wget https://github.com/prometheus/prometheus/releases/latest/download/prometheus-linux-amd64.tar.gz

tar xvf prometheus-linux-amd64.tar.gz

cd prometheus-linux-amd64

Step 2: Configure Prometheus to Scrape Node Exporter

Edit prometheus.yml:

scrape_configs:

- job_name: 'node_exporter'

static_configs:

- targets: ['localhost:9100']

Step 3: Start Prometheus

./prometheus --config.file=prometheus.yml &

Step 4: Install and Start Node Exporter

wget https://github.com/prometheus/node_exporter/releases/latest/download/node_exporter-linux-amd64.tar.gz

tar xvf node_exporter-linux-amd64.tar.gz

cd node_exporter-linux-amd64

nohup ./node_exporter &

Step 5: Install and Configure Grafana

sudo apt install -y grafana

sudo systemctl start grafana-server

sudo systemctl enable grafana-server

5. Results and Achievements

5.1 Intrusion Detection and Anomalies Captured

- Detected multiple network intrusions, including unauthorized access attempts and port scans.

- Notable IP addresses flagged for suspicious activity: (Provide IP addresses or anonymized samples).

- Suricata logs captured instances of brute force attempts targeting SSH and HTTP services.

- Example Suricata log entry:

{"timestamp": "2025-03-02T12:34:56Z", "src_ip": "192.168.1.100", "dest_ip": "192.168.1.200", "event_type": "alert", "alert": {"signature": "SSH Brute Force Attempt", "category": "Attempted User Privilege Gain"}}

5.2 System Performance Insights

- Average network bandwidth consumption: XX Mbps before monitoring, YY Mbps after monitoring setup.

- CPU usage on monitored hosts: XX% peak utilization.

- Grafana dashboards revealed periodic traffic spikes due to scheduled background processes.

6. Automated Alerting using Grafana Webhooks

- Alerts configured for high CPU utilization (>80%), unexpected traffic spikes, and detected intrusions.

- Webhooks triggered notifications via Slack/email.

- Sample alert message:

"timestamp": "2025-03-02T12:34:56Z", "src_ip": "192.168.1.100", "dest_ip": "192.168.1.200", "event_type": "alert", "alert": {"signature": "SSH Brute Force Attempt", "category": "Attempted User Privilege Gain"}}

7. Challenges Faced

- Log Overload: Initial setup produced excessive logs, leading to high storage consumption.

- False Positives: Some benign activities were flagged as intrusions, requiring rule tuning.

- Performance Impact: Suricata increased CPU load; optimization was done by adjusting logging levels.

- Alert Noise: Too many low-priority alerts; refined alerting thresholds to reduce unnecessary notifications.

- Tool Removal & Compatibility Issues: Several tools, including Snort and OpenSearch, had to be removed due to integration challenges and performance constraints.

- Service Start Failures: Some services and tools faced startup issues, requiring troubleshooting and reconfiguration.

- Grafana Integration Issues: Initially attempted to integrate Grafana Loki for log visualization but faced difficulties; replaced it with Prometheus logs and Node Exporter for better compatibility.

8. Managing Services

| Service | Command to Restart |
| --- | --- |
| Suricata | sudo systemctl restart suricata |
| Filebeat | sudo systemctl restart filebeat |
| Elasticsearch | sudo systemctl restart elasticsearch |
| Kibana | sudo systemctl restart kibana |
| Prometheus | sudo systemctl restart prometheus |
| Node Exporter | sudo systemctl restart node_exporter |
| Grafana | sudo systemctl restart grafana-server |

9. Conclusion and Future Improvements

This project successfully implements a Network Monitoring System integrating real-time traffic monitoring, intrusion detection, anomaly detection, performance tracking, and automated reporting. It enhances network security, optimizes performance, and proactively detects threats, making it an effective monitoring solution.

Future Improvements

- Integrating AI-based anomaly detection models.

- Adding more log sources for deeper insights.

- Implementing advanced alerting mechanisms via SMS or Slack.